BILL ACCEPTER

Technical Documentation

Developer-oriented documentation derived from the supplied source archive

Source reviewed: app.py, camera.py, config.py, debug.py, fake_detection.py,
my.py, serial_number.py, stream.py, template_matching.py, requirements.txt,
and the supplied dataset reference images.

# Table of Contents

Table of Contents	2

1. Project Overview	4

1.1 Primary Objective	4

1.2 Supported Denominations	4

1.3 Important Scope Boundary	4

2. System Architecture	5

2.1 Major Components	5

2.2 Execution Model	5

2.3 Data Flow	6

3. Project Structure	6

4. Core Components	7

4.1 MainWindow	7

4.2 Logger	7

4.3 SerialClient	7

4.4 StatusCard	7

5. Main Inspection Workflow	7

5.1 Early-Exit Conditions	8

5.2 Stop Behavior	8

6. Data Structures / Models	8

6.1 Template-Matching Result	8

6.2 Homography	8

6.3 Serial ROI Definitions	8

6.4 Authenticity ROIs	9

7. Communication / Protocol	9

7.1 Serial Transport	9

7.2 Commands Used by Production Workflow	9

7.3 Protocol Limitation	9

8. Computer Vision and OCR	10

8.1 Camera Capture	10

8.2 Denomination Matching	10

8.3 Orientation	10

8.4 Serial Number Extraction	10

8.5 Back-Side Detection	10

9. Authenticity Detection	11

9.1 ROI Localization	11

9.2 Color Analysis	11

9.3 Analysis Pipeline	11

9.4 Decision Logic	11

10. Installation & Setup	12

10.1 Prerequisites	12

10.2 Install Dependencies	12

10.3 Configuration	12

11. Running the Project	12

11.1 Main Application	12

11.2 Standalone Utilities	12

12. Configuration Reference	13

12.1 Production Camera	13

12.2 Matching Parameters	13

12.3 OCR Parameters	13

13. Error Handling & Validation	13

13.1 Serial	13

13.2 Camera	14

13.3 Vision	14

13.4 Global Inspection Exception	14

13.5 Retry / Cancellation	14

14. Important Implementation Details	14

15. Development / Maintenance Notes	15

15.1 Working Directory Dependency	15

15.2 Hardware Port Selection	15

15.3 Serial Response Correlation	15

15.4 Cancellation	15

15.5 Camera Reliability	15

15.6 Authenticity Calibration	15

15.7 Legacy / Experimental Code	15

16. Developer Quick Reference	15

16.1 Production Call Graph	15

16.2 Key Files to Read First	16

16.3 Minimal Operational Checklist	16

Appendix A — Dependency Snapshot	16

Appendix B — Source Audit Notes	17

# 1. Project Overview

Bill Accepter is a desktop banknote inspection application implemented in Python. The primary executable workflow is centered in app.py and combines a PyQt5 user interface, an asynchronous serial-control layer, an IP-camera capture pipeline, SIFT-based banknote identification, dynamic serial-number OCR, and a color-change-based authenticity check for $100 notes.

## 1.1 Primary Objective

The application automates a physical inspection cycle: initialize and position the mechanism, illuminate the banknote, capture an image, determine denomination and orientation, extract the serial number, optionally perform an authenticity test, and finally command the mechanism to move the banknote forward or backward.

## 1.2 Supported Denominations

| Denomination | Template | Authenticity path |
| --- | --- | --- |
| $1 | dataset/1.jpg | Skipped; accepted after denomination detection |
| $5 | dataset/5.jpg | Skipped; accepted after denomination detection |
| $10 | dataset/10.jpg | Skipped; accepted after denomination detection |
| $20 | dataset/20.jpg | Skipped; accepted after denomination detection |
| $100 | dataset/100.jpg | Two-image color/security-feature analysis |

## 1.3 Important Scope Boundary

The code does not implement a general-purpose counterfeit classifier. For non-$100 denominations, the authenticity result is explicitly set to N/A and the note is accepted once denomination detection succeeds. For $100, authenticity is determined by the color-shift analysis in fake_detection.py.

# 2. System Architecture

The production path is orchestrated by MainWindow.run_process(). The Qt GUI remains responsible for user interaction and presentation, while an asyncio event loop runs in a daemon thread for serial I/O and asynchronous orchestration.

## 2.1 Major Components

| Component | Implementation | Responsibility |
| --- | --- | --- |
| GUI / orchestration | app.py | Controls, status, results, logging, process orchestration |
| Serial communication | SerialClient in app.py | Async serial transport, command transmission, response queue |
| Camera capture | take_picture() in app.py | OpenCV capture, frame acquisition, 180° rotation, crop |
| Denomination detection | template_matching.py | SIFT matching against five reference templates and homography validation |
| Serial OCR | serial_number.py | Dynamic ROI localization, perspective normalization, EasyOCR |
| Authenticity | fake_detection.py | Dynamic security ROIs and multi-metric color comparison |
| Configuration | config.py + .env | Camera IP loaded from environment |
| Developer utilities | camera.py, debug.py, my.py, stream.py | Standalone capture, serial control, legacy/experimental matching and RTSP viewing |

## 2.2 Execution Model

Qt's main application thread hosts the UI.

MainWindow creates a dedicated asyncio event loop and starts it in a daemon thread.

GUI actions submit coroutines to that loop with asyncio.run_coroutine_threadsafe().

SerialClient receives raw bytes through asyncio.Protocol.data_received(), decodes them as text, strips whitespace, and pushes responses into an asyncio.Queue.

Computer-vision operations such as camera capture are kept out of the Qt event loop by executing the blocking OpenCV capture function through run_in_executor().

## 2.3 Data Flow

User clicks Connect; the application enumerates serial ports and selects the last returned port.

The serial device is opened at 115200 baud.

Start Inspection resets the UI and schedules run_process().

The mechanism is homed, device state is queried, LEDs are enabled, and the mechanism moves to the inspection position.

The camera returns a cropped image.

serial_number.is_back_side() checks whether the note appears to be the back side.

template_matching.algorithm_with_homography() identifies denomination, estimates 0°/180° orientation, and returns a homography.

The image is rotated when the detected orientation is 180°.

serial_number.extract_serial() maps a reference-coordinate serial ROI into the captured image and runs EasyOCR.

For $100, LEDs are changed and a second image is captured; fake_detection.check_color_shift() compares dynamic security-feature ROIs.

The note is accepted with forward or rejected with back.

# 3. Project Structure

| Path | Role / significance |
| --- | --- |
| app.py | Primary application. Contains UI, serial client, camera capture and the complete 10-step inspection workflow. |
| camera.py | Standalone camera test utility using RTSP and 180° rotation. |
| config.py | Loads .env using python-dotenv and exposes IP. |
| debug.py | Standalone PyQt serial controller for manual hardware commands. |
| fake_detection.py | Dynamic ROI localization and authenticity decision logic. |
| my.py | Earlier feature-based image matching implementation using color/CLAHE features and Euclidean distance. |
| serial_number.py | SIFT homography, dynamic serial ROI extraction, OCR preprocessing and back-side detection. |
| stream.py | Standalone RTSP viewer with reconnect behavior and manual frame capture. |
| template_matching.py | SIFT template matcher for $1/$5/$10/$20/$100 and orientation/homography utilities. |
| dataset/ | Five supplied reference banknote images used as templates/reference imagery. |
| .env | Runtime camera IP configuration. |
| requirements.txt | Pinned Python dependency set. |

Implementation note: app.py is the system entry point. The other modules used directly by the production workflow are template_matching.py, serial_number.py, fake_detection.py, and config.py.

# 4. Core Components

## 4.1 MainWindow

MainWindow owns application state (serial_client, connected, processing), creates the asyncio loop, builds the PyQt5 UI, and orchestrates the inspection lifecycle. Its result cards expose denomination, angle, matching score, serial number, and authenticity.

## 4.2 Logger

Logger is a QObject carrying Qt signals for logs, status, images, progress, step text, and result values. This is the bridge used to update GUI widgets from the asynchronous workflow.

| Signal | Payload | Consumer |
| --- | --- | --- |
| log_signal | text, level | add_log() |
| status_signal | text, status_type | set_status() |
| image_signal | image, target | show_image() |
| progress_signal | integer | set_progress() |
| step_signal | text | set_step() |
| result_signal | key, value | set_result() |

## 4.3 SerialClient

SerialClient subclasses asyncio.Protocol. connection_made() stores the transport and completes a Future; data_received() decodes incoming bytes and enqueues a text response; send() appends a newline and writes the command; ask() waits for one queued response with a default 30-second timeout.

## 4.4 StatusCard

A small reusable Qt frame containing a title and centered value label. It is used for the five inspection result cards.

# 5. Main Inspection Workflow

| Step | Progress | Operation | Key implementation |
| --- | --- | --- | --- |
| 1/10 | 5% | Initialization | 2-second delay; then home/get commands |
| 2/10 | 15% | Illumination | led=1,127 through led=4,127 |
| 3/10 | 25% | Positioning | move=2000; |
| 4/10 | 35% | First image | take_picture(), save cropped1.jpg, display |
| 5/10 | 45% | Back-side check | serial_number.is_back_side(); if true -> back |
| 6/10 | 55% | Denomination | SIFT + RANSAC homography; threshold 0.20 |
| 7/10 | 65% | Orientation | 180° image rotation if required |
| 8/10 | 75% | Serial number | Dynamic ROI + perspective warp + EasyOCR |
| 9/10 | 85% | $100 authenticity | LED change + second image + color analysis |
| 10/10 | 95%→100% | Final mechanism action | forward if accepted, otherwise back |

## 5.1 Early-Exit Conditions

Camera capture failure causes the workflow to return without accepting the note.

Back-side detection causes the mechanism to receive back and exits the workflow.

No valid denomination match causes back and exits.

For $100, failure to capture the second image produces UNKNOWN authenticity and rejection.

Unexpected exceptions are logged as errors and the UI is returned to a non-processing state.

## 5.2 Stop Behavior

Stop sets self.processing to False and updates the UI, but it does not cancel an already-running coroutine or abort an in-flight serial ask(). The workflow checks self.processing after the initial sleep, while later stages rely on completion/exception handling.

# 6. Data Structures / Models

## 6.1 Template-Matching Result

match_template_sift() returns a dictionary containing:

score — weighted combination of inlier ratio and normalized inlier count.

good_matches — number of matches passing Lowe's ratio test.

inliers — RANSAC inlier count.

inlier_ratio — inliers / good_matches.

homography — 3×3 projective transformation matrix, or None.

template_keypoints / input_keypoints — SIFT keypoints.

good_match_objects / inlier_mask — diagnostic matching data.

## 6.2 Homography

The production vision stack treats the homography as the transformation from reference-image coordinates to the current camera image. That single geometric relationship is reused to locate denomination-dependent serial ROIs and the $100 authenticity ROIs.

## 6.3 Serial ROI Definitions

| Bill | Right ROI (x1,x2,y1,y2) | Left ROI (x1,x2,y1,y2) |
| --- | --- | --- |
| $1 | (1140,1610,180,280) | None |
| $5 | (1070,1560,460,570) | (40,550,120,210) |
| $10 | (1140,1620,450,580) | (90,600,160,240) |
| $20 | (1160,1610,480,580) | (60,570,150,230) |
| $100 | (1260,1720,500,630) | (40,600,170,290) |

These coordinates are defined in the reference-image coordinate system. They are transformed by the homography before extraction.

## 6.4 Authenticity ROIs

| ROI | Reference coordinates | Normalized output |
| --- | --- | --- |
| $100 security region | (1330,1800,600,750) | 470 × 150 |
| Bell region | (1030,1330,500,710) | 300 × 210 |

# 7. Communication / Protocol

## 7.1 Serial Transport

| Property | Observed implementation |
| --- | --- |
| Transport | pyserial-asyncio via serial_asyncio.create_serial_connection() |
| Baud rate | 115200 |
| Port selection | ports[-1] after serial.tools.list_ports.comports() |
| Command encoding | UTF-8 bytes |
| Command terminator | send() appends '\n' |
| Response handling | decode(errors='ignore').strip() then asyncio.Queue |
| Default response timeout | 30 seconds per ask() |

## 7.2 Commands Used by Production Workflow

| Command | Purpose |
| --- | --- |
| home | Move mechanism to home position |
| get | Read device state; invoked twice during initialization |
| led=1,127; … led=4,127; | Enable four illumination channels at value 127 |
| move=2000; | Move to inspection position |
| back | Return/reject banknote |
| forward | Accept banknote / move forward |
| led=1,0; … led=4,0; | Turn off selected illumination channels for $100 authenticity capture |

The debug.py utility additionally exposes move=<n>, move+<n>, move-<n>, roler1=<speed>,<time>;, roler2=<state>,<time>;, and led=<number>,<value>; controls. These are manual developer controls rather than part of the main inspection path.

## 7.3 Protocol Limitation

The application associates one queued response with each ask() call but does not implement a command identifier, response correlation ID, framing parser beyond newline-based serial writes, or retry policy. data_received() treats each callback's decoded payload as one response.

# 8. Computer Vision and OCR

## 8.1 Camera Capture

app.py uses the configured IP to build an HTTP video endpoint: http://<IP>:8080/video. OpenCV reads up to 30 frames and stops at the first successful frame. The frame is rotated 180° and cropped using x1=40, x2=w, y1=50, y2=h-230. The cropped image is then saved as cropped1.jpg.

Implementation note: camera.py and stream.py use the RTSP endpoint rtsp://<IP>:8080/h264.sdp instead. This means the production application and the standalone camera utilities are configured for different capture URLs.

## 8.2 Denomination Matching

Five templates are loaded at module import from dataset/1.jpg, dataset/5.jpg, dataset/10.jpg, dataset/20.jpg, and dataset/100.jpg.

Each template is converted to grayscale and processed with CLAHE.

SIFT is configured with nfeatures=5000 and template descriptors are precomputed.

The input image is processed in the same way and matched against every template using BFMatcher with NORM_L2.

Lowe's ratio test uses 0.70.

At least 10 good matches are required.

cv2.findHomography(..., cv2.RANSAC, 5.0) estimates the geometric transform.

At least 8 inliers and an inlier ratio of at least 0.20 are required.

The final score is 0.6 × inlier_ratio + 0.4 × min(inliers/100, 1.0).

The highest-scoring denomination is selected before threshold validation.

## 8.3 Orientation

estimate_orientation() transforms the template corners using H, averages the top and bottom horizontal edge vectors, computes their direction with arctan2, and classifies the result as 0° or 180° based on proximity to those directions.

## 8.4 Serial Number Extraction

serial_number.py loads dataset/100.jpg as a reference image and caches its SIFT descriptors.

The caller normally passes the homography already calculated by template_matching.py.

The denomination-specific ROI is transformed from reference coordinates to camera coordinates.

The transformed quadrilateral is perspective-warped into a rectangular ROI.

The ROI is converted to grayscale, upscaled 2×, Gaussian-blurred, and converted into adaptive-threshold and Otsu variants.

EasyOCR reads each variant using an alphanumeric allowlist.

The result is normalized and searched for the pattern [A-Z]{1,2}[0-9]{8}[A-Z].

An exact match is returned immediately; otherwise the longest partial OCR result is returned as Not Found (Partial: ...), or Not Found.

## 8.5 Back-Side Detection

is_back_side() runs EasyOCR on the original image and a 180° rotation. If any recognized text contains TRUST, the image is classified as back side.

# 9. Authenticity Detection

Authenticity is only invoked for denomination $100. The application changes the LED state, captures a second image, corrects its orientation if the first match was 180°, and compares two dynamic security-feature ROIs.

## 9.1 ROI Localization

fake_detection.py computes a reference→current homography independently for each image using SIFT, Lowe ratio 0.70, RANSAC threshold 5.0, at least 10 good matches, and at least 8 inliers. It then transforms the $100 and Bell ROIs and extracts fixed-size perspective-normalized images.

## 9.2 Color Analysis

| Metric / threshold | Implementation |
| --- | --- |
| MIN_COLOR_CHANGE_RATIO = 0.08 | At least 8% of pixels must exceed the chromaticity-change threshold. |
| MIN_CHROMATICITY_DIFF = 0.025 | Mean normalized RGB/chromaticity difference threshold and local-change pixel threshold. |
| MIN_HUE_DIFF = 5.0 | Mean circular HSV hue difference. |
| MIN_SATURATION_DIFF = 8.0 | Mean HSV saturation difference. |
| MIN_HISTOGRAM_DISTANCE = 0.08 | Bhattacharyya distance between hue histograms. |
| MIN_POSITIVE_TESTS = 2 | Minimum aggregate positive tests for the fallback decision. |

## 9.3 Analysis Pipeline

LAB normalization with CLAHE on the L channel is applied to both ROI images.

Normalized chromaticity is calculated as r/(r+g+b), g/(r+g+b), b/(r+g+b).

Mean chromaticity difference is calculated.

HSV hue and saturation differences are calculated; hue distance is circular.

The ratio of pixels whose chromaticity difference exceeds 0.025 is calculated.

A normalized hue histogram is compared using Bhattacharyya distance.

Each ROI is also split into a 3×4 grid and block-level chromaticity differences are measured.

## 9.4 Decision Logic

For each ROI, five boolean tests are evaluated: chromaticity, local change, hue, saturation, and histogram. The total number of positive tests across the two ROIs is counted out of 10.

primary_signal_100 = local_change_test and chromaticity_test

primary_signal_bell = local_change_test and chromaticity_test

strong_signal = primary_signal_100 or primary_signal_bell

supporting_signal = secondary_signal_100 and secondary_signal_bell

is_real = strong_signal or (

positive_tests >= MIN_POSITIVE_TESTS

and supporting_signal

)

The function returns True for REAL and False for FAKE. The optional threshold argument is retained for compatibility but ignored by the current implementation.

# 10. Installation & Setup

## 10.1 Prerequisites

The repository provides pinned Python dependencies but does not declare a Python interpreter version. Therefore a compatible Python version must be selected experimentally against the pinned package set; the source itself does not establish an exact runtime version.

## 10.2 Install Dependencies

python -m venv .venv

.venv\Scripts\activate

python -m pip install --upgrade pip

pip install -r requirements.txt

On POSIX systems, activate the environment with . .venv/bin/activate. The exact package pins are taken from requirements.txt; notable runtime packages include PyQt5 5.15.11, OpenCV 4.13.0.92, numpy 2.4.4, pyserial 3.5, pyserial-asyncio 0.6, python-dotenv 1.2.2, torch 2.12.0, and the transitive/explicit OCR stack represented by the supplied requirements.

## 10.3 Configuration

IP=192.168.70.139

config.py calls load_dotenv() and exposes IP=os.getenv('IP'). app.py then constructs the production camera URL from this value.

Implementation note: The .env file contains a machine/network address. For deployment, keep environment-specific values outside source control and replace the example with the actual camera address.

# 11. Running the Project

## 11.1 Main Application

python app.py

The main process creates QApplication, instantiates MainWindow, shows the window, and enters the Qt event loop.

## 11.2 Standalone Utilities

| Command | Purpose |
| --- | --- |
| python camera.py | Capture a frame from the RTSP camera and save frame.jpg. |
| python stream.py | Open an RTSP viewer, reconnect after loss, and save frames with Space. |
| python debug.py | Open a manual serial-controller GUI. |
| python template_matching.py | Run standalone template matching against test.jpg and optionally visualize homography/matches. |

my.py is a standalone earlier matching implementation and is not imported by app.py. It expects the dataset directory and performs a feature-distance voting process.

# 12. Configuration Reference

## 12.1 Production Camera

| Setting | Value / source | Effect |
| --- | --- | --- |
| IP | .env → config.py | Camera host address |
| Production URL | http://<IP>:8080/video | VideoCapture source used by app.py |
| Crop | x=40..w, y=50..h-230 | Removes fixed borders/regions from captured frame |
| Rotation | 180° | Normalizes camera mounting orientation |

## 12.2 Matching Parameters

| Constant | Value |
| --- | --- |
| SIFT_FEATURES | 5000 |
| LOWE_RATIO | 0.70 |
| RANSAC_THRESHOLD | 5.0 |
| MIN_GOOD_MATCHES | 10 |
| MIN_INLIERS | 8 |
| MIN_INLIER_RATIO | 0.20 |
| MIN_SCORE | 0.20 |

## 12.3 OCR Parameters

| Setting | Value |
| --- | --- |
| SIFT_FEATURES | 5000 |
| LOWE_RATIO | 0.70 |
| RANSAC_THRESHOLD | 5.0 |
| MIN_GOOD_MATCHES | 10 |
| MIN_INLIERS | 8 |
| OCR scale | 2× |
| Serial pattern | [A-Z]{1,2}[0-9]{8}[A-Z] |
| EasyOCR allowlist | 0123456789ABCDEFGHIJKLMNOPQRSTUVWXYZ |
| EasyOCR model directory | models |

# 13. Error Handling & Validation

## 13.1 Serial

No serial ports: logs an error and returns to Disconnected.

Connection exceptions: logged and connection state reset.

ask() timeout: logs a warning and returns None.

send() without a transport raises RuntimeError.

## 13.2 Camera

VideoCapture failure returns None.

Up to 30 frames are attempted before declaring capture failure.

run_process() treats a failed first or second capture as a rejection/failure path as applicable.

## 13.3 Vision

Template matching rejects results with insufficient homography, score, inliers, or inlier ratio.

Serial homography rejects insufficient good matches or inliers.

Authenticity homography rejects insufficient features/matches/inliers and invalid/out-of-image ROIs.

## 13.4 Global Inspection Exception

run_process() wraps the inspection sequence in a broad exception handler, logs the exception, sets the step to Inspection failed, and uses finally to restore button states and connection status.

## 13.5 Retry / Cancellation

There is no general retry mechanism in app.py. The standalone stream.py utility does reconnect after RTSP loss, but that behavior is not reused by take_picture(). The Stop button changes processing state but does not cancel the asyncio task.

# 14. Important Implementation Details

template_matching.py loads all five template images and computes their SIFT descriptors at import time. The application therefore depends on the current working directory containing dataset/.

serial_number.py initializes EasyOCR's Reader at module import. Model loading can therefore affect application startup time.

fake_detection.py caches the $100 reference image's SIFT features after first use; serial_number.py similarly caches its reference descriptors.

The production workflow reuses the denomination homography for serial-number ROI extraction, reducing redundant geometric matching for that stage.

fake_detection.py independently calculates a homography for each authenticity image rather than receiving the first-stage homography from app.py.

The $100 authenticity path intentionally changes illumination before the second capture, so the comparison is based on two controlled illumination states.

The score shown in the UI is the composite SIFT/homography score, not OCR confidence or authenticity confidence.

The UI displays '$' before denomination values, while the underlying value is numeric.

The serial result is returned as text and may be an exact serial, a partial OCR result, or Not Found.

The application currently emits the Connected status signal twice after a successful serial connection; this is harmless visually but redundant.

# 15. Development / Maintenance Notes

## 15.1 Working Directory Dependency

Several paths are relative (dataset/100.jpg, dataset/<value>.jpg, models, cropped1.jpg, cropped2.jpg). Launching app.py from a different working directory can therefore break template/reference loading or change output locations.

## 15.2 Hardware Port Selection

async_connect() automatically selects ports[-1]. This is convenient in a controlled machine but is not deterministic across systems. A future production hardening step should identify the intended controller by VID/PID, serial number, description, or explicit configuration.

## 15.3 Serial Response Correlation

The current protocol assumes responses arrive in the expected order. If unsolicited messages, multi-line responses, or delayed responses are introduced, the single shared asyncio.Queue can associate a response with the wrong command. A framed protocol or command/response correlation scheme would make this more robust.

## 15.4 Cancellation

The Stop control should ideally cancel the scheduled inspection task and explicitly bring the mechanism to a safe hardware state. The current implementation only changes the UI state variable.

## 15.5 Camera Reliability

take_picture() creates a new VideoCapture object for every image and has no retry/reconnect loop. The existing stream.py demonstrates a reconnect pattern that could be integrated if continuous operation requires better camera resilience.

## 15.6 Authenticity Calibration

The authenticity thresholds are fixed constants in fake_detection.py. The source comments indicate that they should be calibrated with real/fake samples. Any production change to these thresholds should be validated against representative capture conditions, because illumination and camera behavior directly affect the measured color differences.

## 15.7 Legacy / Experimental Code

my.py contains an older feature-vector matching approach based on 3D color histograms, HSV histograms, CLAHE pixels, image partitioning, and Euclidean distance. It is not part of app.py's import graph. camera.py, debug.py, and stream.py are similarly standalone tools. They should be treated as development utilities unless explicitly promoted into the production path.

# 16. Developer Quick Reference

## 16.1 Production Call Graph

MainWindow.start_process()

└── run_process()

├── SerialClient.ask("home")

├── SerialClient.ask("get")

├── SerialClient.ask("led=...")

├── SerialClient.ask("move=2000;")

├── take_picture()

├── serial_number.is_back_side()

├── template_matching.algorithm_with_homography()

├── serial_number.extract_serial(..., H)

│     └── get_dynamic_serial_roi(..., H)

│           └── warp_roi()

│     └── optimize_serial_ocr()

├── [if $100]

│     ├── take_picture()

│     └── fake_detection.check_color_shift()

│           ├── get_dynamic_rois()

│           └── analyze_roi() × 2

└── SerialClient.ask("forward" | "back")

## 16.2 Key Files to Read First

app.py — complete application lifecycle and integration points.

template_matching.py — denomination detection and homography semantics.

serial_number.py — serial ROI geometry and OCR behavior.

fake_detection.py — authenticity ROI geometry, metrics, and decision logic.

config.py / .env — camera endpoint configuration.

requirements.txt — pinned environment dependencies.

## 16.3 Minimal Operational Checklist

Confirm the camera is reachable at the configured IP.

Confirm the serial controller appears as a port and is the port selected by the current ports[-1] logic.

Run the application from the project root so relative dataset paths resolve.

Verify the five reference images exist in dataset/.

Allow EasyOCR model initialization/download if models are not already present.

Check the execution log before tuning thresholds; it exposes SIFT scores, inliers, OCR candidates, and authenticity diagnostics.

# Appendix A — Dependency Snapshot

| Package | Pinned version |
| --- | --- |
| easyocr | 1.7.2 |
| filelock | 3.29.0 |
| fsspec | 2026.4.0 |
| ImageIO | 2.37.3 |
| Jinja2 | 3.1.6 |
| lazy-loader | 0.5 |
| MarkupSafe | 3.0.3 |
| mpmath | 1.3.0 |
| networkx | 3.6.1 |
| ninja | 1.13.0 |
| numpy | 2.4.4 |
| opencv-python | 4.13.0.92 |
| opencv-python-headless | 4.13.0.92 |
| packaging | 26.2 |
| pillow | 12.2.0 |
| pyclipper | 1.4.0 |
| PyQt5 | 5.15.11 |
| PyQt5-Qt5 | 5.15.2 |
| PyQt5_sip | 12.18.0 |
| pyserial | 3.5 |
| pyserial-asyncio | 0.6 |
| python-bidi | 0.6.10 |
| python-dotenv | 1.2.2 |
| PyYAML | 6.0.3 |
| scikit-image | 0.26.0 |
| scipy | 1.17.1 |
| setuptools | 81.0.0 |
| shapely | 2.1.2 |
| sympy | 1.14.0 |
| tifffile | 2026.5.2 |
| torch | 2.12.0 |
| torchvision | 0.27.0 |
| typing_extensions | 4.15.0 |

# Appendix B — Source Audit Notes

The supplied archive contained 17 entries: nine Python/source files, one requirements file, one .env file, five JPEG dataset/reference images, and the dataset directory entry. The documentation above distinguishes the production execution path from standalone/debug/legacy utilities rather than treating every file as an active runtime component.

Where behavior could not be established from the source itself (for example, the exact firmware protocol implementation behind the serial commands or the physical meaning of individual security features), the documentation deliberately describes the observed software behavior instead of inferring undocumented hardware semantics.